# Solar Radiation Forecasting System

Technical Report: Dataset, Preprocessing, Feature Engineering & Ensemble

# 1. Dataset & Preprocessing

## 1.1 Data Source

The dataset is sourced from the NASA POWER (Prediction of Worldwide Energy Resources) API — hourly meteorological and solar radiation data for Lahore, Pakistan (31.56°N, 74.35°E).

File: lahore_hourly_filled.csv

Date range: January 2018 – October 2025

Temporal resolution: 1 hour

Approximate size: ~68,000 hourly records

## 1.2 Raw Columns

| Column | Description | Unit |
| --- | --- | --- |
| SolarRadiation | Target variable — Global Horizontal Irradiance (GHI) | W/m² |
| ClearSkyRadiation | Theoretical maximum radiation under clear sky | W/m² |
| DirectRadiation | Direct Normal Irradiance (DNI) | W/m² |
| DiffuseRadiation | Diffuse Horizontal Irradiance (DHI) | W/m² |
| SolarZenith | Sun angle from vertical (≥90° = night) | degrees |
| Temperature | Air temperature at 2 m | °C |
| HumiditySpecific | Specific humidity | g/kg |
| HumidityRelative | Relative humidity | % |
| Pressure | Surface pressure | kPa |
| WindSpeed | Wind speed at 10 m | m/s |
| WindDirection | Wind direction at 10 m | degrees |

## 1.3 Preprocessing Steps

The following steps are applied in training.py before any model training:

Parse datetime index and sort chronologically.

df.apply(pd.to_numeric, errors='coerce') — coerce all columns to numeric, handling any string artifacts from the CSV.

Drop rows where SolarRadiation is NaN.

Night hours are intentionally kept (not filtered) — models learn that SolarZenith ≥ 90° implies zero radiation.

A derived feature is then computed:

ClearnessIndex = SolarRadiation / ClearSkyRadiation, clipped to [0, 1].

Zero-safe: when ClearSkyRadiation = 0, ClearnessIndex = 0 (avoids division by zero).

Interpretation: values near 1 = clear sky; values near 0 = heavy cloud cover.

# 2. Train/Test Split & Leakage Prevention

## 2.1 Chronological 80/20 Split

A strict chronological split is used — no shuffling at any stage:

Training set:  ~2018 → mid-2023  (80% of records)

Test set:      ~mid-2023 → Oct 2025  (20% of records)

Code: split_idx = int(len(df_day) * 0.8)

## 2.2 Leakage Prevention Measures

| Measure | How it is enforced |
| --- | --- |
| Scaler fitting | All StandardScalers are fit exclusively on training data, then transform() is applied to test data. |
| Lag feature safety | Lag features use .shift(lag) — only past values are used; no future values enter the feature window. |
| Sequence assignment | Sequences spanning the train/test boundary are assigned to the set that contains their target timestamp, not their window start. |
| Validation split | LSTM/CNN-LSTM use validation_split=0.1 on the training set — Keras takes the last 10% of training chronologically (no shuffle). |
| Early stopping | EarlyStopping(patience=15, restore_best_weights=True) — stops on validation loss without touching test data. |
| Learning rate decay | ReduceLROnPlateau(patience=5) — adapts on validation, not test performance. |

# 3. Feature Engineering

## 3.1 Tree Models (XGBoost, Random Forest) — 41 Features

17 base features plus 24 autoregressive lag features:

| Category | Features | Count |
| --- | --- | --- |
| Raw time | hour, month, day_of_year | 3 |
| Cyclical time | hour_sin, hour_cos (period 24) month_sin, month_cos (period 12) doy_sin, doy_cos (period 365) | 6 |
| Solar geometry | SolarZenith, ClearSkyRadiation | 2 |
| Weather | Temperature, HumiditySpecific, HumidityRelative, Pressure, WindSpeed, WindDirection | 6 |
| Autoregressive | lag_1, lag_2, …, lag_24 (past 24 hours of SolarRadiation) | 24 |

Rationale for cyclical encoding: Raw hour=23 and hour=0 are numerically far apart but temporally adjacent. Sine/cosine encoding wraps this into a continuous circle, preventing the model from treating midnight as discontinuous.

Rationale for SolarZenith: A physical constraint — when Zenith ≥ 90° the sun is below the horizon and radiation must be zero. This acts as the single most discriminative binary feature for night prediction.

## 3.2 Sequence Models (LSTM, CNN-LSTM, TFT, TCN) — 15 Features × 24 Timesteps

14 features (no raw time integers, no explicit lag columns) fed as a 24-step sliding window. The sequence window itself carries the temporal history equivalent to lag features.

| Category | Features |
| --- | --- |
| Cyclical time | hour_sin, hour_cos, month_sin, month_cos, doy_sin, doy_cos |
| Solar geometry | SolarZenith, ClearSkyRadiation |
| Weather | Temperature, HumiditySpecific, HumidityRelative, Pressure, WindSpeed, WindDirection |
| Target (in seq) | SolarRadiation (past values included in input window) |

Input tensor shape: (N, 24, 15) — N samples × 24 timesteps × 15 features.

All features are scaled with a StandardScaler fit on training data. A separate scaler_y is used for the target column to allow inverse_transform of predictions back to W/m².

# 4. Ensemble Weight Formation

## 4.1 Training-Derived Weights (training.py)

After all models are trained, weights are assigned proportionally to their Test R² scores:

w_i  =  max(0, R²_i)  /  Σ_j  max(0, R²_j)

This ensures better-performing models receive higher influence, and any model with negative R² is excluded (weight floored to 0). The weights are saved to saved_models/ensemble_weights.pkl.

## 4.2 Production Weights (frontend/config.py)

Two manually tuned weight sets exist at runtime, depending on which external forecast API is used:

| Model | NASA POWER Weight | Open-Meteo Weight | Rationale |
| --- | --- | --- | --- |
| XGBoost | 0.30 | 0.20 | NASA data format closely matches XGBoost training data; down-weighted when using real-time API |
| Random Forest | 0.25 | 0.18 | Same reasoning as XGBoost |
| LSTM | 0.08 | 0.12 | Sequence models valued more when real-time weather context is available |
| CNN-LSTM | 0.07 | 0.10 | Same as LSTM |
| TFT | 0.18 | 0.25 | Transformer attention handles uncertainty better with real-time data |
| TCN | 0.12 | 0.15 | Causal convolutions adapt well to recent signal patterns |
| Total | 1.00 | 1.00 | — |

## 4.3 Per-Hour Calibration Multipliers

After the weighted blend, a per-hour scalar is applied to correct for systematic timing biases:

NASA POWER calibration: Hours 0–6 and 17–23 → ×0.0 (hard zero for night). Hours 7–16 → ×1.0.

Open-Meteo calibration: More granular ramp curve to counteract underestimation of the late-afternoon value by Open-Meteo. Hour 17 carries a ×10.0 multiplier, reflecting a known API bias at dusk.

Final Ensemble(h)  =  Cal(h)  ×  Σ_i  [ w_i  ×  ŷ_i(h) ]

# 5. Model Accuracy

## 5.1 Individual Model Performance

| Model | Train R² | Test R² | Test RMSE (W/m²) | Test MAE (W/m²) |
| --- | --- | --- | --- | --- |
| XGBoost | 0.9981 | 0.9933 | 22.62 | 9.69 |
| Random Forest | 0.9963 | 0.9933 | 22.77 | 9.88 |
| CNN-LSTM | 0.9891 | 0.9896 | 28.22 | 17.01 |
| LSTM | 0.9876 | 0.9873 | 31.30 | 20.19 |
| TFT | 0.9959 | 0.9472 | 63.69 | 28.58 |
| TCN | 0.9936 | 0.9461 | 64.38 | 28.83 |

## 5.2 Key Observations

Tree models (XGBoost, Random Forest) dominate on test metrics with Test R² = 0.9933 and MAE below 10 W/m². This is why they receive the highest ensemble weights (0.25–0.30).

CNN-LSTM outperforms plain LSTM: the Conv1D layers extract local temporal patterns before the BiLSTM layers process longer-range dependencies, yielding ~3 W/m² lower RMSE.

TFT and TCN show overfitting: Train R² ~0.995 vs Test R² ~0.947. The gap indicates these models did not generalize as well on the 2023–2025 test window, possibly due to fewer training epochs or architectural under-regularization.

Ensemble advantage: A single R² number cannot capture the robustness benefit. On extreme or out-of-distribution days (unusual cloud patterns, storm events), the ensemble hedges against catastrophic failure of any single model by blending diverse predictions.

## 5.3 Why the Ensemble Is Still Preferred

Even though XGBoost/RF alone achieve the highest individual Test R², the ensemble is used in production for several reasons:

Variance reduction: Combining models with different architectures (tree, recurrent, convolutional, attention) reduces prediction variance on unseen dates outside the 2018–2025 training window.

API calibration: The production ensemble integrates real-time API forecasts (NASA POWER / Open-Meteo) as a soft prior, correcting for weather anomalies that historical ML models cannot anticipate.

Hour-level reliability: The calibration multipliers explicitly correct known systematic biases (night zeros, dusk underestimation) that individual models may still predict as small non-zero values due to noisy training data.